| Assembly {reference} | Simulation of JAW[1] Date: Friday, December 31, 2021 Designer: Solidworks, Setup by Isaac Wax Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions Error! Bookmark not defined. Contact Information 5 Mesh information 5 Sensor Details 5 Resultant Forces 6 Beams 6 Study Results 7 Conclusion 10. |
| --- | --- |
| # Description Static optimization simulation for claw in horizontal fixture on ROV. |  |

# Assumptions

# Model Information Model name: JAW[1] Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet15; Solid Body; Mass:0.00774607 kg Volume:7.04181e-06 m^3 Density:1,100.01 kg/m^3 Weight:0.0759115 N; G:\My Drive\ROV--2022\MECHAICAL\Claw\Claw 2.3\JAW[1].SLDPRT Dec 31 10:55:03 2021.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (G:\My Drive\ROV--2022\MECHAICAL\Claw\Claw 2.3).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; Epoxy, Unfilled. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Tensile strength:; 2.8e+07 N/m^2. Compressive strength:; 1.04e+08 N/m^2. Elastic modulus:; 2.415e+09 N/m^2. Poisson's ratio:; 0.35. Mass density:; 1,100 kg/m^3.; SolidBody 1(Fillet15)(JAW[1]). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 63.4159; -0.00435513; -9.65982; 64.1474. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; 7 kgf..

# Contact Information The goal is only to look at that jaw component and not at the gears, so it is constrained rigidly throughout the 6 mm pin-hole

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points for High quality mesh; 16 Points. Element Size; 1.91745 mm. Tolerance; 0.0958726 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 15020. Total Elements; 9076. Maximum Aspect Ratio; 6.9571. % of elements with Aspect Ratio < 3; 98.9. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 63.4159; -0.00435513; -9.65982; 64.1474. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.00134599; -0.0239432; 0.0137557; 0.0276461. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 5.406e+02N/m^2 Node: 6010; 1.041e+07N/m^2 Node: 881. JAW[1]-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 1; 9.537e-01mm Node: 11920. JAW[1]-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 2.936e-07 Element: 2505; 3.381e-03 Element: 6683. JAW[1]-Static 1-Strain-Strain1. Name; Type. Displacement1{1}; Deformed shape. JAW[1]-Static 1-Displacement-Displacement1{1}.

# Conclusion

This part seems adequate at 7Kgf in the horizontal mounting position, but may want to add more aggressive filets to the high stress areas.